TOCI V2021 Condition Assessment

Survey123 User Guide

2.10.2021

This page intentionally left blank.

Table of Contents

Introduction	1

Logging into Survey123	1

Using Survey123 TOCI V2021 Condition Assessment	3

Accessing Inspections through the Inbox	4

Map View	6

Completing the Condition Assessment Form	8

Grid Name, Circuit and Circuit FLOC (Circuit Section)	8

Is Circuit Information Correct in the Field?	9

Circuit	9

Circuit FLOC (Circuit Section)	9

Voltage	9

FLOC	9

Is the Structure Number correct on the structure?	10

Are You Able to Proceed with this Inspection?	11

Inspection Date	11

Inspection Type	11

Inspector Name	11

Yes, Proceed with this Inspection Steps	12

Structure Type (Steel Lattice/Tower)	12

Structure Type (Pole)	12

Structure Attachments	13

Structure Attachment Details	14

Data Collection Questions	15

Is there a FLOC Number on the Structure?	15

Is there a Buddy Pole?	15

What is attached to Buddy Pole?	15

Describe the Area Surrounding the Structure within 100 feet.	15

Is the Structure within an Enclosed Area?	15

Type of Footing	16

Indicate the Equipment Type(s) and Attachments on the Span.	16

Does the Foundation have a ground to the structure?	16

Identify the appearance of the Footings/Foundations	16

Indicate the Equipment Type(s) and attachments on the Structure	17

Switch Type	17

How many Transmission Lightning Arresters are Installed on this Structure?	17

Do the Insulators Connect to the Structure using a Hook?	17

Is Transmission Line hose/Bird Deterrent present?	17

If Shield Wire are on a 500kV Structure, is the Shield Wire:	18

Indicate the Type(s) of Conductor Splice present on the Span.	18

How Many Conductor Splices?	18

Indicate the Type(s) of Jumper Splice present on the Span?	18

Bulk: Are Line Splices at least 100 feet from point of support?	18

How Many Transmission DOWN guys are installed?	18

How Many Transmission SPAN guys are installed?	18

Are Span/Arm Guys installed parallel to the line conductor?	19

Do Guy pass near/through Distribution Circuits?	19

Do Guys have Insulation between Structure and Ground?	19

Guy Type and Attachments	19

Is the 4th Wire Bonded to the Structure?	19

Main Structure and Supporting Members Questions	20

Does Bird Nest/Wildlife debris require notification?	20

Does Main Structure require notification (excluding Crossarm/arm/x-braces)?	20

Do Supporting Members (braces/crossarms/hardware) require notification (excluding guys/guying)?	21

Guys/Guying	22

Equipment Questions	26

Does Equipment Require Notification?	26

Does the Riser Require a Notification?	27

Does the Equipment Brackets or Braces Require a Notification?	28

Insulators / Hardware / Wires / Misc Questions	29

Do insulators require notification?	30

Do visible Bonds/Bonding and/or copper grounding require notification?	30

Do Jumpers require notification?	31

Do Dampers require notification?	32

Does Line Hardware require notification?	32

Do Wires or Splices require notification?	33

Does 3rd Party Cable require notification?	33

Does Suspension Insulator Plum require notification?	34

Does Pilot Wire/Open Wire require notification?	34

Does Conductor Spacers require notification?	35

Grounding /Other Questions	36

Footings / Foundation Questions	37

Vegetation Management Questions	38

Transmission ROW Questions	39

PRIORITY 1	39

Notifications	40

Priority	40

Find and Fix?	41

Unable to Inspect Steps	42

Are You Able to Proceed with this Inspection?	42

Unable to Inspect Category	42

Photos	44

Photo Type	44

Photos	44

Work Status	44

Submit the Completed Inspection	45

Saving as Draft	46

Signing Out of Survey123	47

Downloading the Latest Version of TOCI 2021 Condition Assessment Form	48

# Introduction

The TOCI Ground Inspection Survey123 Form is used to document ground inspections and create notifications for any conditions found.

NOTE: These screenshots were taken on a laptop. The screens may look slightly different from what you see on your iPad.

## Logging into Survey123

| Select on the Survey 123 icon to launch the app. |  |
| --- | --- |
| When the app opens, Select Sign In. |  |
| In the Sign in Screen, enter in your ArcGIS ID in the Username field. Enter the temporary password. Select Sign in. |  |
| You will be prompted to set a new password. In the Old Password field, enter in the default password. In the New Password field, enter in a new password. The new password must: - Be a minimum of eight (8) characters. - Contain at least one uppercase letter. - Contain at least one lowercase letter. - Contain at least one number. Select Change Password. |  |
| You will be prompted to create a security question and answer. You can choose a different question. Select the dropdown arrow and Select the question you want to use. Your answer should be just one word. It is not case-sensitive. Select Ok. You are now logged in. NOTE: Once you are logged into Survey123 on this iPad, you will remain logged in even if you close the app or turn off the iPad. |  |

## Using Survey123 TOCI V2021 Condition Assessment

| When you log into Survey123 for the first time, you will not see any surveys on your device because none have been downloaded yet. - Select Get Surveys. |  |
| --- | --- |
| Select the icon for the TOCI V2021 Condition Assessment survey. The app will download the TOCI V2021 Condition Assessment survey form. Once the download is complete, click Ok. The Download Surveys screen displays and the icon next to the TOCI V2021 Condition Assessment form appears. |  |
| Select the back arrow < in the upper left corner next to Download Surveys to return to the home screen. |  |
| The TOCI V2021 Condition Assessment survey now appears in My Surveys. Select the TRANS V2021 Condition Assessment icon. |  |

The TOCI V2021 Condition Assessment form displays with the Collect and Inbox folders.

## Accessing Inspections through the Inbox

| All inspections assigned for completion will appear in the Inbox. Select the Inbox icon at the bottom of the screen. |  |
| --- | --- |
| The first time you log in, the Inbox will be empty. Select Refresh in the lower right corner. |  |
| A list of all the inspections that have been assigned to you will be displayed in the List view. There are a few options to open the inspection form: Select on the structure in List View. NOTE: When there is more than one structure, the Sort and Search features are available for you to use. The Inspection form opens. |  |
| NOTE: Once you begin an inspection, the structure will disappear off the list in your Inbox and off the map. You also have the option to view the assigned inspections in Map view. |  |

### Map View

| - Select Map to view the structures on a map. The World Street map opens and the structures display with a blue diamond along with the structure numbers when zoomed in. NOTE: If you have cellular service and are connected to the internet, new inspections may be pushed to your device and will automatically be added to your list. You will need to tap Refresh at the bottom right of the screen. |  |  |
| --- | --- | --- |
| Structures assigned to you for inspection appear as blue diamonds. To zoom in on the map, you can pinch and drag or double-click + or -- on the map. |  |  |
| There are five other map view options: World Imagery World Topography USA Topo Maps Open Street Map Open Street Map Humanitarian Select the map gallery icon. |  |  |

- Tap on a structure (either in the list or on the map) to begin the inspection.

The Condition Assessment form opens.

NOTE: Once you begin an inspection, the structure will disappear off the list in your Inbox and off the map.

## Completing the Condition Assessment Form

- Fields with black text are read-only.

- Fields marked with an * are required. You will not be able to submit the inspection form if any required fields are left empty.

| The Apple Maps Navigation link opens up the map for driving directions. You can access images via GRViewer (log in is required). ### Grid Name, Circuit and Circuit FLOC (Circuit Section) These fields will auto populate based on the original data provided. |  |
| --- | --- |

| ### Is Circuit Information Correct in the Field? Select Yes or No. If No is selected, the Circuit field displays. ### Circuit Type or select the correct Circuit name from the list. NOTE: Selecting the Circuit name narrows down the Circuit FLOC (Circuit Section) name. ### Circuit FLOC (Circuit Section) Select the down-arrow to select the Circuit FLOC (Circuit Section). |  |
| --- | --- |
| ### Voltage The voltage pre-populates based on the original circuit provided. |  |
| ### FLOC Select the refresh button to populate the FLOC number. |  |

| ### Is the Structure Number correct on the structure? Select Yes, No, or N/A (Tower) If No is selected, you will receive a message as below and the “Are you able to proceed with this inspection” question displays If No, go to the Unable to Inspect Steps |  |
| --- | --- |
| These fields will auto populate and are read only. Mile Tower Number Object Type Object Subtype High Fire Tier Latitude and Longitude fields are read-only. These are populated from cGIS. |  |
| ### Are You Able to Proceed with this Inspection? Select the appropriate answer: Yes – inspection questions will display. Proceed with the Yes, Able to Proceed with Inspection steps. No – if Incorrect Number, No Access, Obstructed, Incorrect Grid, Non-Transmission, or Pole Removed. Proceed to the No, Unable to Inspect Steps. |  |
| ### Inspection Date Select the Refresh button to populate the current date and time. |  |
| ### Inspection Type Select the appropriate type of inspection that you are completing. |  |
| ### Inspector Name Select / enter the Inspector’s first name. |  |

## Yes, Proceed with this Inspection Steps

| ### Structure Type (Steel Lattice/Tower) Select the appropriate Structure Type: Steel Lattice/Tower Pole - see below. #### Structure Subtype Select the appropriate structure subtype for Steel Lattice / Tower. If Other is selected, provide details in the Other Structure Type field. |  |
| --- | --- |
| ### Structure Type (Pole) If Structure type is pole, select the appropriate structure subtype. If Other is selected, provide details in the Other Structure Type field. |  |

| #### Main Pole Material Type Displays when Structure Type = Pole. Select the appropriate main pole material type. If Other is selected, provide details in the Other Main Pole Material field. |  |
| --- | --- |
| #### Supporting Members Material Type Displays when Structure Type = Pole. Select all that apply for Supporting Members Material Type. If Other is selected, provide details in the Other Supporting Members Material Type field. |  |
| ### Structure Attachments Select the appropriate structure attachment(s) from the list. Select all that apply. |  |

## Structure Attachment Details

Identify which attachments are on the structure (select all that apply).  Other questions will display and shall be answered based on the selections made in this section.

## Data Collection Questions

| ### Is there a FLOC Number on the Structure? Select Yes or No. |  |
| --- | --- |
| ### Is there a Buddy Pole? Select Yes or No. If Yes, the What is attached to Buddy Pole question displays. ### What is attached to Buddy Pole? Select all that apply. |  |
| ### Describe the Area Surrounding the Structure within 100 feet. Select all that apply. |  |
| ### Is the Structure within an Enclosed Area? Select Yes or No. If Yes, the Enclosed Area Type question displays. #### Enclosed Area Type Select all that apply. |  |

| ### Type of Footing Select all that apply. If Other, the Other Type of Footing field displays. If Unable to Determine, the Unable to Determine Type of Footing question displays. #### Other Type of Footing Provide a description of the other footing type. #### Unable to Determine Type of Footing Select all that apply to describe why you are not able to determine footing type. |  |
| --- | --- |
| ### Indicate the Equipment Type(s) and Attachments on the Span. Select all that apply. If Switch is selected, the Switch Type question displays. #### Switch Type Select the appropriate switch type. |  |
| ### Does the Foundation have a ground to the structure? Select Yes or No. |  |
| ### Identify the appearance of the Footings/Foundations Select all that apply. |  |
| ### Indicate the Equipment Type(s) and attachments on the Structure Select the appropriate equipment type. If Switch, then the Switch Type question displays. If Risers/Potheads/Arresters, then the How many Transmission Lightning Arresters are installed on the this structure question displays. |  |
| ### Switch Type Identify the type of switch on the structure. |  |
| ### How many Transmission Lightning Arresters are Installed on this Structure? Select the appropriate number. |  |
| ### Do the Insulators Connect to the Structure using a Hook? Displays when Porcelain Insulator is selected. Select Yes or No. |  |
| ### Is Transmission Line hose/Bird Deterrent present? Select Yes or No. If Yes, the Transmission Line hose/Bird Deterrent Type question displays. #### Transmission Line hose/Bird Deterrent Type Select all that apply. |  |
| ### If Shield Wire are on a 500kV Structure, is the Shield Wire: Select all that apply. |  |
| ### Indicate the Type(s) of Conductor Splice present on the Span. Select all that apply. |  |
| ### How Many Conductor Splices? Select the appropriate number. |  |
| ### Indicate the Type(s) of Jumper Splice present on the Span? Select all that apply. If Not Present or Unable to Determine is selected, the Bulk: Are Line Splices at least 100 feet from point of support question displays. |  |
| ### Bulk: Are Line Splices at least 100 feet from point of support? Select Yes or No. |  |
| ### How Many Transmission DOWN guys are installed? Select the appropriate number. |  |
| ### How Many Transmission SPAN guys are installed? Select the appropriate number. |  |
| ### Are Span/Arm Guys installed parallel to the line conductor? Select Yes or No. If Yes, the next question displays. #### How Many Span/Arm Guys are installed parallel to the line conductor? Select the appropriate number. |  |
| ### Do Guy pass near/through Distribution Circuits? Select Yes or No. |  |
| ### Do Guys have Insulation between Structure and Ground? Select Yes or No. If Yes, the Guy Insulation Type question displays. #### Guy Insulation Type Select all that apply. |  |
| ### Guy Type and Attachments Select all that apply. If Other, the Other Guy Type and Attachments field displays. #### Other Guy Type and Attachments Provide a description of other guy type and attachments. |  |
| ### Is the 4th Wire Bonded to the Structure? Select Yes or No. |  |

## Main Structure and Supporting Members Questions

| ### Does Bird Nest/Wildlife debris require notification? Select Yes or No or Duplicate. If Yes, provide Notification Reasons. If Duplicate, indicate the Notification Number. #### Bird Nest/Wildlife Debris Notification Reasons. Select all that apply. #### Bird Nest/Wildlife Debris Notification Number Enter the Notification Number. |  |
| --- | --- |
| ### Does Main Structure require notification (excluding Crossarm/arm/x-braces)? Select Yes or No or Duplicate. If Yes, provide Notification Reasons. You must also create a Notification. If Duplicate, indicate the Notification Number. #### Main Structure Notification Reasons Select all that apply. #### Bird Nest/Wildlife Debris Notification Number Enter the Notification Number. |  |

| ### Do Supporting Members (braces/crossarms/hardware) require notification (excluding guys/guying)? Select Yes, No or Duplicate. If Yes, provide Notification Reasons. You must also create a Notification. If Duplicate, indicate the Notification Number. #### Supporting Members Notification Reasons Select all that apply. #### Supporting Members Notification Number Enter the Notification Number. |  |
| --- | --- |

## Guys/Guying

| #### Do DOWN guys/guying require notification (excluding anchors)? Select Yes, No, Unable to Determine, or Duplicate. If Yes, then indicate the DOWN Guys Notification Reason(s). You must also create a Notification. If Unable to Determine, then indicate the unable to determine reason(s). If Duplicate, indicate the Notification Number. #### DOWN Guys Notification Reasons Select all that apply. #### Unable to Determine Down Guy Notification Select all that apply. #### DOWN Guy Notification Number Enter the Notification Number. |  |
| --- | --- |

| #### Do SPAN guys/guying require notification (excluding anchors)? Select Yes, No, or Unable to Determine. If Yes, then indicate the SPAN Guys Notification Reason(s). You must also create a Notification. If Unable to Determine, then indicate the unable to determine reason(s). If Duplicate, indicate the Notification Number. #### SPAN Guys/Guying Notification Reasons Select all that apply. #### Unable to Determine Down Guy Notification Select all that apply. #### Span Guy Notification Number Enter the Notification Number. |  |
| --- | --- |

| #### Do Anchors Require Notification? Select Yes, No, or Unable to Determine. If Yes, select the Anchors Notification Reason(s). You must also create a Notification. If Unable to Determine, then indicate the unable to determine reason(s). If Duplicate, indicate the Notification Number. #### Anchors Notification Reasons Select all that apply. #### Unable to Determine Anchor Notification Select all that apply. #### Anchors Notification Number Enter the Notification Number. |  |
| --- | --- |
| #### Do Guys/Anchors require notification due to Clearance (<5ft) from non-SCE metallic objects? If Yes, answer the next question. You must also create a Notification. If Unable to Determine, then indicate the unable to determine reason(s). If Duplicate, indicate the Notification Number. #### Is the Object in Direct Contact with Guys/Anchors? Select Yes or No. #### Unable to Determine Guy/Anchor Clearance Select all that apply. #### Guy/Anchor Clearance Notification Number Enter the Notification Number. |  |

## Equipment Questions

| ### Does Equipment Require Notification? Select the appropriate answer. If Yes, you must indicate the Notification Reason(s). You must also create a Notification. If Duplicate, indicate the Notification Number. #### Equipment Type and Attachments Notification Reasons Select the appropriate reason(s) for the notification. #### Equipment Type and Attachments Notification Number Enter the Notification Number. |  |
| --- | --- |

| ### Does the Riser Require a Notification? Select the appropriate answer. If Yes, you must indicate the Notification Reason(s). You must also create a Notification. If Unable to Determine, then indicate the unable to determine reason(s). If Duplicate, indicate the Notification Number. #### Riser Notification Reasons Select the appropriate reason(s) for the notification. #### Unable to Determine Riser Notification Select all that apply. #### Riser Notification Number Enter the Notification Number. |  |
| --- | --- |

| ### Does the Equipment Brackets or Braces Require a Notification? Select the appropriate answer. If Yes, you must indicate the Notification Reason(s). You must also create a Notification. If Duplicate, indicate the Notification Number. #### Equipment Bracket or Braces Notification Reasons Select the appropriate reason(s) for the notification. #### Equipment Bracket or Braces Notification Number Enter the Notification Number. |  |
| --- | --- |

## Insulators / Hardware / Wires / Misc Questions

| ### Do insulators require notification? Select the appropriate answer. If yes, provide Notification Reasons. You must also create a notification. If Duplicate, indicate the Notification Number. #### Insulators Notification Reasons Select the appropriate reason(s) for the notification. #### Insulators Notification Number Enter the Notification Number. |  |
| --- | --- |
| ### Do visible Bonds/Bonding and/or copper grounding require notification? Select the appropriate answer. If yes, provide Notification Reasons. You must also create a notification. If Duplicate, indicate the Notification Number. #### Bonding Notification Reasons Select the appropriate reason(s) for the notification. #### Bonding Notification Number Enter the Notification Number. |  |
| ### Do Jumpers require notification? Select the appropriate answer. If yes, provide Notification Reasons. You must also create a notification. If Duplicate, indicate the Notification Number. #### Jumpers Notification Reasons Select the appropriate reason(s) for the notification. #### Jumpers Notification Number Enter the Notification Number. |  |

| ### Do Dampers require notification? Select the appropriate answer. If yes, provide Notification Reasons. You must also create a notification. If Duplicate, indicate the Notification Number. #### Dampers Notification Reasons Select the appropriate reason(s) for the notification. #### Dampers Notification Number Enter the Notification Number. |  |
| --- | --- |
| ### Does Line Hardware require notification? Select the appropriate answer. If yes, provide Notification Reasons. You must also create a notification. If Duplicate, indicate the Notification Number. #### Hardware Notification Reasons Select the appropriate reason(s) for the notification. #### Hardware Notification Number Enter the Notification Number. |  |
| ### Do Wires or Splices require notification? Select the appropriate answer. If yes, provide Notification Reasons. You must also create a notification. If Duplicate, indicate the Notification Number. #### Wires or Splices Notification Reasons Select the appropriate reason(s) for the notification. #### Wires or Splices Notification Number Enter the Notification Number. |  |
| ### Does 3rd Party Cable require notification? Select the appropriate answer. If yes, provide Notification Reasons. You must also create a notification. If Duplicate, indicate the Notification Number. #### 3rd Party Cable Notification Reasons Select the appropriate reason(s) for the notification. #### 3rd Party Cable Notification Number Enter the Notification Number. |  |
| ### Does Suspension Insulator Plum require notification? Select the appropriate answer. If yes, provide Notification Reasons. You must also create a notification. If Duplicate, indicate the Notification Number. #### Suspension Insulator Plum Notification Reasons Select the appropriate reason(s) for the notification. #### Suspension Insulator Plum Notification Number Enter the Notification Number. |  |
| ### Does Pilot Wire/Open Wire require notification? Select the appropriate answer. If yes, provide Notification Reasons. You must also create a notification. If Duplicate, indicate the Notification Number. #### Pilot Wire/Open Notification Reasons Select the appropriate reason(s) for the notification. #### Suspension Insulator Plum Notification Number Enter the Notification Number. |  |
| ### Does Conductor Spacers require notification? Select the appropriate answer. If yes, provide Notification Reasons. You must also create a notification. If Duplicate, indicate the Notification Number. #### Conductor Spacers Notification Reasons Select the appropriate reason(s) for the notification. #### Conductor Spacers Notification Number Enter the Notification Number. |  |

## Grounding /Other Questions

| #### Does Shield Wire require notification? Select the appropriate answer. If yes, provide Notification Reasons. You must also create a notification. If Duplicate, indicate the Notification Number. #### Shield Wire Notification Reasons Select the appropriate reason(s) for the notification. #### Shield Wire Spacers Notification Number Enter the Notification Number. |  |
| --- | --- |
| #### Does FRC require notification? Select Yes or No. If Yes, select the Reason(s), enter notification comments, and create a Notification. If Duplicate, indicate the Notification Number. #### FRC Notification Reasons Select the appropriate reason(s) for the notification. #### FRC Notification Number Enter the Notification Number. |  |

## Footings / Foundation Questions

| #### Does the Footing/Base require a notification? Select Yes or No. If Yes, select the Reason(s), enter notification comments, and create a Notification. If Unable to Determine, provide reason. If Duplicate, indicate the Notification Number. #### Footing/Base Notification Reasons Select the appropriate reason(s). #### Unable to Determine Footing/Base Notification Select the appropriate reason(s). #### Footing/Base Notification Number Enter the Notification Number. |  |
| --- | --- |

## Vegetation Management Questions

| #### Is vegetation trimming or removal required to prevent contact above and/or below transmission conductor, equipment, or guying? Select Yes ,No or Duplicate. If Yes, select the Notification Reason(s), and create a Notification. If Duplicate, provide the notification number. #### Vegetation Management Notification Distance Select the appropriate reason(s). #### Vegetation Management Notification Number Ener the Notification Number. |  |
| --- | --- |

## Transmission ROW Questions

| #### Is a notification required for brush clearing? Select Yes or No. If Yes, select the appropriate notification Reason(s), and create a Notification. #### Brush Clearing Notification Reasons Select the appropriate reason(s). |  |
| --- | --- |

## PRIORITY 1

If a priority 1 was identified, follow the existing procedures.

| #### Was a Priority 1 Notification identified during this inspection? Select Yes or No. If Yes, provide the CMS Notification Number. #### CMS Created Notification Number Enter the P1 notification number. |  |
| --- | --- |

| ## Notifications Follow the steps below to create notifications. NOTE: A notification must be created for each condition. #### Notification Needed, If Any? This field will auto populate based on the answers provided pertaining to notification required. If notifications are needed, the system will indicate the number of notifications that you will need to create for this inspection. ### Priority Select the appropriate priority code (P2 or P3) for each notification that needs to be created. |  |
| --- | --- |
| Create the notification by selecting the appropriate options based on the condition: Object Type (Item Object Part) will be pre-populated based on the data for the structure. Object Part (Item Object Piece) Elevation (Coding) Condition (Item Damage Piece) Action Required (Activity Code) Review the problem statement created to ensure it matches the condition identified. ### Find and Fix? Select Yes or No. If Yes, enter the Cause of Damage and Action Taken Codes. The Circuit Name and Circuit FLOC (Circuit Section) will auto populate. #### Notifications Needed Comments Provide a detail description of the condition identified. You must create a separate Notification for each condition found. Select the + icon to create an additional Notification(s) and follow the steps above. |  |

# Unable to Inspect Steps

| ## Are You Able to Proceed with this Inspection? - Select No if unable to inspect. |  |
| --- | --- |
| ### Unable to Inspect Category Select the appropriate Unable to Inspect Category for why you are not able to inspect the structure. Incorrect Structure Number ** No Access** Obstructed View Non-Transmission Removed Structure (Not in field/active in SAP) Duplicate Record Poor Photo Quality (Aerial Ground Footage) ** These categories require additional information. See below. |  |

| Incorrect Structure Number If Incorrect Structure Number, provide the structure number in the field and use the Collect feature to complete the inspection for the structure number indicated in the field. |  |
| --- | --- |
| No Access Select the appropriate reason for No Access: Property Owner – provide Responsible Patrolman name Brush Overgrown – Must create a Notification for weed/abatement brush clearing. See Notification s |  |

## Photos

At least two photos are required to be attached to the inspection:

Full Structure

FLOC (Structure #)

Additionally, for each Notification created, you must provide a photo(s) clearly depicting the condition identified.

| ### Photo Type Indicate the appropriate photo type being attached to the Notification. ### Photos - Take a photo via the iPad camera so that the photo(s) are saved in your iPad Photos folder. Select the folder icon to access the appropriate file where the photo is located. To add additional photos, select the Plus icon and enter any necessary photo comments. |  |
| --- | --- |

## Work Status

The Work Status for each inspection will be automatically populated based on the answers you provided for the inspection and cannot be changed.

# Submit the Completed Inspection

The Submit button appears as a checkmark in the bottom right corner.

- Select the checkmark  (bottom right) to submit the completed inspection. If your device is online, the inspection will be sent and saved in your Sent folder.

Once the inspection has been submitted, it will disappear from your Inbox.

REMEMBER: You cannot submit a form if required fields have been left blank. You will be prompted to fill out each required field.

| If your device is online and all the fields have been filled out, you have two options for submitting: You can “Send Later” if you are not ready to send yet and will appear in your Outbox. The form will be sent immediately if you click, “Send Now” and will appear in your Sent folder. NOTE: If your device is offline (no cell service), you will only have the option to save the form to your device and will appear in your Outbox. IMPORTANT: When your iPad goes back online be sure to go to your Outbox to submit the completed inspections. |  |
| --- | --- |

## Saving as Draft

| Select at the top of the screen to close and save draft. The Confirm Close message displays. Select Save this survey in Drafts. |  |
| --- | --- |
| The inspection is saved in the “Drafts” folder. NOTE: If you refresh your Inbox, the saved draft inspection will re-appear in your Inbox since the work status is still in “Assigned” status. |  |

# Signing Out of Survey123

Follow the steps below to sign out of Survey123.

| Check your Survey screen. You should only see the Collect, Inbox, and Sent folders. If you see the Drafts folder that means you have work saved that needs to be completed, submitted, and synced. Open the Drafts folder and resolve all saved forms located there. If you see the Outbox folder that means that you have completed inspections that needs to be submitted when cellular service resumes. To clear the Outbox folder when cellular service resumes, you will need to click the Outbox and click Send All button to submit. |  |
| --- | --- |
| - Return to the My Surveys screen. Select the menu (three bars in the upper right corner). Select Sign out. |  |

# Downloading the Latest Version of TOCI 2021 Condition Assessment Form

The Survey123 application is being updated frequently.

IMPORTANT: You should download the Survey once a day, preferably first thing in the morning before you start work so that you are always using the most up-to-date form.

| - Return to the My Surveys screen. - Select the menu (three bars in the upper right corner) to display the menu. - Select Download Surveys. The Download Surveys window displays. |  |
| --- | --- |
| To locate the TOCI Ground Inspection form: - In the Search field, type “toci”. The TOCI V2021 Condition Assessment Form appears at the top of the list. - Select the TOCI V2021 Condition Assessment link to download it. |  |
| When the survey is downloaded, select OK. |  |
| Select the back arrow (left side) next to Download Surveys to return to the My Surveys screen. |  |